SCOPE 156 ЛИ

Sofia, 04.06.2024

ELKABEL JSC, BURGAS

TEST LABORATORY

Management and laboratory address:

8000 Burgas, 15 Odrin Str.

To perform testing of:

| Type of the scope: flexible |  |  |  |
| --- | --- | --- | --- |
| № | Tested products | Type of test / characteristic | Testing methods (standard/ validated method) |
| 1 | 2 | 3 | 4 |
| 1. 1 | Power cables and conductors with thermoplastic insulation of rated voltages up to and including 450/750 V | 1. Measurement of radial thickness of insulation / sheath | БДС EN 60811-100 БДС EN 60811-201 БДС EN 60811-202 БДС EN 50396 cl. 4.1, cl. 4.2, cl. 4.3 ГОСТ 12177 cl. 3.4.7 |
|  |  | 2. Measurement of constructive dimensions | БДС EN 60811-100 БДС EN 60811-203 БДС 6259 cl. 2.4, cl. 2.6, cl. 2.8; БДС EN 50396 cl. 4.4; BS 6004 cl. 15.2 |
|  |  | 3. Mechanical properties of insulation / sheath before and after ageing | БДС EN 60811-100 БДС EN 60811-501 БДС EN 60811-401 cl. 4.1 a), b); ГОСТ 25018 |
|  |  | 4. Loss of mass of insulation / sheath | БДС EN 60811-100 БДС EN 60811-409 |
|  |  | 5. Pressure test at high temperature of insulation / sheaths | БДС EN 60811-100 БДС EN 60811-508 |
|  |  | 6. Test at low temperature | БДС EN 60811-100 БДС EN 60811-504 БДС EN 60811-505 БДС EN 60811-506 |
|  |  | 7. Test for resistance of insulations and sheaths to cracking (heat shock test) | БДС EN 60811-100 БДС EN 60811-509 |
|  |  | 8. Thermal stability of insulation / sheath | БДС EN 60811-100 БДС EN 60811-405 |
|  |  | 9. Electrical resistance of conductors, screens and armours | БДС 2374 БДС EN 60228, Annex A ГОСТ 7229 |
|  |  | 10. Test of cable and/or cores under alternating voltage | БДС 2406 cl. 1; БДС EN 50395 cl. 6, cl. 7, cl. 10.3 ГОСТ 2990 cl. 4.1 BS 6004, Annex E |
|  |  | 11. Electrical resistance of insulation and/or sheath | БДС 1986 БДС EN 50395 cl. 8 ГОСТ 3345 |
|  |  | 12. Compatibility of cable components in thermal ageing | БДС EN 60811-100 БДС EN 60811-501 БДС EN 60811-401 cl. 4.1 c) |
|  |  | 13. Water absorption of PVC insulation | БДС 2406 cl. 1, cl. 2 БДС EN 50395 cl. 9 |
|  |  | 14. Surface resistance of sheath | БДС EN 50395, cl. 11 |
|  |  | 15. Mineral oil immersion tests for sheaths at increased temperature | БДС EN 60811-100 БДС EN 60811-404 |
|  |  | 16. Test of single insulated conductor or cable under fire conditions | БДС EN 60332-1-2 |
|  |  | 17. Test of small single insulated conductor or cable under fire conditions | БДС EN 60332-2-2 |
| 2. | Power cables with cross-linked insulation of rated voltages up to and including 450/750 V | 1. Measurement of radial thickness of insulation / sheath | БДС EN 60811-100 БДС EN 60811-201 БДС EN 60811-202 БДС EN 50396 cl. 4.1, cl. 4.2, cl. 4.3 |
|  |  | 2. Measurement of constructive dimensions | БДС EN 60811-100 БДС EN 60811-203 БДС EN 50396 cl. 4.4 |
|  |  | 3. Mechanical properties of insulation / sheath before and after ageing | БДС EN 60811-100 БДС EN 60811-501 БДС EN 60811-401 cl. 4.1 a), b) |
|  |  | 4. Test at low temperature | БДС EN 60811-100 БДС EN 60811-504 БДС EN 60811-505 БДС EN 60811-506 |
|  |  | 5. Electrical resistance of conductors, screens and armours | БДС EN 60228, Annex A |
|  |  | 6. Test of cable and/or cores under alternating voltage | БДС EN 50395 cl. 6, cl. 7, cl. 10.3 BS 7211, Annex E |
|  |  | 7. Electrical resistance of insulation and/or sheath | БДС EN 50395 cl. 8 |
|  |  | 8. Compatibility of cable components in thermal ageing | БДС EN 60811-100 БДС EN 60811-501 БДС EN 60811-401 cl. 4.1 c) |
|  |  | 9. Hot set test of insulation / sheath | БДС EN 60811-100 БДС EN 60811-507 |
|  |  | 10. Mineral oil immersion tests for sheaths at increased temperature | БДС EN 60811-100 БДС EN 60811-404 |
|  |  | 11. Surface resistance of sheath | БДС EN 50395 cl. 11 |
|  |  | 12. Test of single insulated conductor or cable under fire conditions | БДС EN 60332-1-2 |
|  |  | 13. Test of small single insulated conductor or cable under fire conditions | БДС EN 60332-2-2 |
| 3. | Power cables with extruded insulation of rated voltages up to 20/36 kV | 1. Measurement of radial thickness of insulation / sheath | БДС EN 60811-100 БДС EN 60811-201 БДС EN 60811-202 БДС HD 605 S3 cl. 2.1.1, cl. 2.1.11.1 ГОСТ 12177 cl. 3.4.7 |
|  |  | 2. Measurement of constructive dimensions | БДС 6259 cl. 2.4, cl. 2.6, cl. 2.7.2.2, cl. 2.8 БДС EN 60811-100 БДС EN 60811-203 БДС HD 605 S3 cl. 2.1.11.1, cl. 2.1.11.2, cl. 2.1.4.2, cl. 2.1.13.2 ІEC 60502-1 cl. 16.6, cl. 16.7 ІEC 60502-2 cl. 17.6, cl. 17.7 |
|  |  | 3. Mechanical properties of insulation / sheath before and after ageing | БДС EN 60811-100 БДС EN 60811-501 БДС EN 60811-401 cl. 4.1 a), b) ГОСТ 25018 |
|  |  | 4. Loss of mass of insulation / sheath | БДС EN 60811-100 БДС EN 60811-409 |
|  |  | 5. Pressure test at high temperature for insulation / sheaths | БДС EN 60811-100 БДС EN 60811-508 |
|  |  | 6. Test at low temperature | БДС EN 60811-100 БДС EN 60811-504 БДС EN 60811-505 БДС EN 60811-506 |
|  |  | 7. Test for resistance of insulations and sheaths to cracking (heat shock test) | БДС EN 60811-100 БДС EN 60811-509 |
|  |  | 8. Thermal stability of insulation / sheath | БДС EN 60811-100 БДС EN 60811-405 |
|  |  | 9. Electrical resistance and resistivity of conductors, screens and armours | БДС 2374 БДС HD 605 S3 cl. 3.1.1, cl. 3.1.2, cl. 3.1.4 БДС EN 60228, Annex A IEC 60502-1 cl. 15.2 IEC 60502-2 cl. 16.2; BS 6724 Annex B.5 ГОСТ 7229 |
|  |  | 10. Test of cable and/or cores under alternating voltage | БДС 2406 cl. 1 БДС HD 605 S3 cl. 3.2.1, cl. 3.2.2, cl. 3.2.5 IEC 60502-1 cl. 15.3.1, cl. 15.3.2 а); cl. 15.3.3; cl. 15.3.4; cl. 15.3.5; cl. 17.4 IEC 60502-2 cl. 17.9, cl. 18.2.9, cl. 18.3.4 BS 6485 Annex A1 BS 6622 cl. 17.8.2, cl. 18.16 БДС EN 50395 cl. 10.3 ГОСТ 2990 cl. 4.1 NF C 33-209 cl. 6.6 |
|  |  | 11. Electrical resistance and resistivity of insulation and/or sheath | БДС 1986 БДС HD 605 S3 cl. 3.3.1 IEC 60502-1 cl. 17.2, cl. 17.3 IEC 60502-2 cl. 18.3.2, cl. 18.3.3; BS 6485, Annex A2 NF C 33-209 cl. 6.3; ГОСТ 3345 |
|  |  | 12. Compatibility of cable components in thermal ageing | БДС EN 60811-100 БДС EN 60811-501 БДС EN 60811-401 cl. 4.1 c) |
|  |  | 13. Hot set test of insulation / sheath | БДС EN 60811-100 БДС EN 60811-507 |
|  |  | 14. Shrinkage test of insulation / sheath | БДС EN 60811-100 БДС EN 60811-502 БДС EN 60811-503; БДС HD 605 S3 cl. 2.4.4.1; BS 6724, Annex H |
|  |  | 15. Surface resistance of sheath | БДС HD 605 S3 cl. 3.4 |
|  |  | 16. Water absorption of PVC insulation | БДС 2406 cl. 1, cl. 2 БДС EN 60811-100 БДС EN 60811-402 cl. 4.3; |
|  |  | 17. Water penetration test | БДС 2581 cl. 3.32; БДС HD 605 S3 cl. 2.4.9.3; IEC 60502-2 cl. 19.24, Annex F |
|  |  | 18. Resistivity of semiconducting screens | БДС HD 605 S3 cl. 3.9.1; IEC 60502-2 cl. 18.2.10, Annex D; BS 6622, Annex J |
|  |  | 19. Mineral oil immersion tests for sheaths at increased temperature | БДС EN 60811-100 БДС EN 60811-404 |
|  |  | 20. Heating cycle test | БДС 2581:86 cl. 3.15; БДС HD 620 S3 10A cl. 3.3.1.4; БДС HD 620 S3 10C cl. 3.3.1.4; БДС HD 620 S3 10J cl. 3.3.4.2; БДС HD 632 S3 Part 1 cl. 12.4.6; БДС HD 605 S3 cl. 3.8.1; cl. 3.8.2; IEC 60502-2 cl. 18.2.7; BS 6622 cl. 20.6; |
|  |  | 21. Bending test | БДС 2581 cl. 3.12; БДС HD 605 S3 cl. 2.4.1.6; IEC 60502-2 cl. 18.2.4; BS 6622 cl. 20.3 |
|  |  | 22. Measurement of Shore D hardness of PE sheath | БДС HD 605 S3 cl. 2.2.1 |
|  |  | 23. Stress cracking resistance for PE sheath | БДС EN 60811-100 БДС EN 60811-406 БДС HD 605 S3 cl. 2.4.21 |
|  |  | 24. Irregularities in the insulation and semiconducting screens | БДС HD 605 S3 cl. 2.1.10.2; cl. 2.1.10.5 |
|  |  | 25. Measurement of dielectric losses | IEC 60502-2 cl. 18.2.6; БДС HD 605 S3 cl. 3.11; БДС 6927 |
|  |  | 26. Measurement of partial discharges | БДС EN 60885-3; БДС 13382; БДС HD 605 S3 cl. 3.10 |
|  |  | 27. Test of single insulated conductor or cable under fire conditions | БДС EN 60332-1-2 |
| 4. | Power cables with extruded insulation of rated voltages up to 64/110 kV | 1. Measurement of radial thickness of insulation / sheath | БДС EN 60811-100 БДС EN 60811-201 БДС EN 60811-202 |
|  |  | 2. Measurement of constructive dimensions | БДС 6259 cl. 2.7.2.2, cl. 2.8 БДС EN 60811-100 БДС EN 60811-203 ІEC 60840 cl. 10.7 |
|  |  | 3. Mechanical properties of insulation / sheath before and after ageing | БДС EN 60811-100 БДС EN 60811-501 БДС EN 60811-401 cl. 4.1 a), b) |
|  |  | 4. Loss of mass of insulation / sheath | БДС EN 60811-100 БДС EN 60811-409 |
|  |  | 5. Pressure test at high temperature for insulation / sheaths | БДС EN 60811-100 БДС EN 60811-508 |
|  |  | 6. Test at low temperature | БДС EN 60811-100 БДС EN 60811-505 БДС EN 60811-506 |
|  |  | 7. Test for resistance of insulations and sheaths to cracking (heat shock test) | БДС EN 60811-100 БДС EN 60811-509 |
|  |  | 8. Electrical resistance and resistivity of conductors, screens and armours | БДС 2374; БДС HD 605 S3 cl. 3.1.1, cl. 3.1.2, cl. 3.1.4 ІEC 60840 cl. 10.5; БДС HD 632 S3 Part 1 cl. 10.5 |
|  |  | 9. Test of cable and/or cores under alternating voltage | БДС HD 632 S3 Part 1 cl. 9.3, cl. 12.4.1; IEC 60840 cl. 9.3; cl. 12.4.1 BS 7912 cl. 9.3, cl. 12.11 |
|  |  | 10. Compatibility of cable components in thermal ageing | БДС EN 60811-100 БДС EN 60811-501 БДС EN 60811-401 cl. 4.1 c) |
|  |  | 11. Hot set test of insulation / sheath | БДС EN 60811-100 БДС EN 60811-507 |
|  |  | 12. Shrinkage test of insulation / sheath | БДС EN 60811-100 БДС EN 60811-502 БДС EN 60811-503 |
|  |  | 13. Water penetration test | БДС HD 632 S3 Part 1, Annex E IEC 60840, Annex E BS 7912 cl. 14 |
|  |  | 14. Resistivity of semiconducting screens | БДС HD 632 S3 Part 1 cl. 12.4.9; Annex D IEC 60840, Annex D |
|  |  | 15. Heating cycle test | БДС HD 632 S3 Part 1 cl. 12.4.6; IEC 60840 cl. 12.4.6; BS 7912 cl. 12.9 |
|  |  | 16. Bending test | БДС HD 632 S3 Part 1 cl. 12.4.3; IEC 60840 cl. 12.4.3; BS 7912 cl. 12.5 |
|  |  | 17. Irregularities in the insulation and the semiconducting screens | БДС HD 605 S3 cl. 2.1.10.2, 2.1.10.5 |
|  |  | 18. Measurement of dielectric losses | БДС 6927; IEC 60840 cl. 12.4.5; БДС HD 632 S3 Part 1 cl. 12.4.5 |
|  |  | 19. Measurement of partial discharges | БДС EN 60885-3 БДС 13382 БДС HD 605 S3 cl. 3.10 |
| 5. | Bare conductors for overhead power lines | 1. Measurement of constructive dimensions | БДС 6259 cl. 2.4; БДС 16651 cl. 4.6; БДС EN 50182 cl. 6.4.2, cl. 6.4.4; IEC 61089 cl. 6.6.1.3 |
|  |  | 2. Electrical resistance and resistivity | БДС 2374 БДС 1133, Annex 2 IEC 60468, Routine Method BS 215-1, Annex A BS 215-2, Annex A |
|  |  | 3. Mechanical properties | БДС 1133 Annex 2 БДС 7454 БДС 17002 cl. 3.8 БДС EN 50189 cl. 11.3, cl. 11.4 БДС EN ISO 6892-1 BS 215-1, Annex A BS 215-2, Annex A |
|  |  | 4. Mass of the conductor | БДС 1133 cl. 4.7; БДС 17002 cl. 3.5; БДС EN 50182 cl. 6.4.6; ІEC 61089 cl. 6.6.3 BS 215-1, Annex A BS 215-2, Annex A |
| 6. | Cu and Al conductors for cables and wires | 1. Measurement of constructive dimensions | БДС 6259 cl. 2.4; БДС HD 605 S3 cl. 2.1.4.2, cl. 2.1.13.2; БДС 16651 cl. 4.6 |
|  |  | 2. Electrical resistance and resistivity | БДС 2374; БДС HD 605 S3 cl. 3.1.1; БДС EN 60228, Annex A; IEC 60468, Routine Method; |
|  |  | 3. Mechanical properties | БДС 7454; БДС EN ISO 6892-1 |

Flexible scope: Implementing a new version of standards/documents or standards/ documents replacing them is allowed. An updated list of standards/documents and their dated versions is provided by laboratory.