SCOPE 9 ЛК

BULGARIAN INSTITUTE OF METROLOGY

General Directorate National Centre of Metrology

Mechanical Measurements Department

Mass Measuring Instruments Calibration Laboratory

Management address: 1797 Sofia, 52B G.M. Dimitrov Blvd.

laboratory address: 8010 Burgas, r.c. Slaveykov, 25 Prof. Yakim Yakimov Str.

To perform calibration of:

| Scope of accreditation: fixed |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| № | Type of measuring instrument | Measured quantuty, measurement unit | Measurement range | Measure ment uncertainty | Calibration method |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 1. | Weights class F1 | Mass, kilogram kg and its submultiples and multiples | from 1 mg to 500 mg | from 0,012 mg to 0,014 mg | ЛКСИМ-М-02:2021 |
|  |  |  | from 1 g to 500 g | from 0,014 mg to 0,12 mg |  |
|  |  |  | from 1 kg to 5 kg | from 2 mg to 5 mg |  |
|  |  |  | from 10 kg to 20 kg | from 10 mg to 20 mg |  |
| 2. | Weights class F2 | Mass, kilogram kg and its submultiples and multiples | from 1 mg to 500 mg | from 0,012 mg to 0,014 mg | ЛКСИМ-М-02:2021 |
|  |  |  | from 1 g to 500 g | from 0,020 mg to 0,14 mg |  |
|  |  |  | from 1 kg to 5 kg | from 3,0 mg to 17 mg |  |
|  |  |  | from 10 kg to 20 kg | from 39 mg to 69 mg |  |
| 3. | Weights class M1 | Mass, kilogram kg and its submultiples and multiples | from 1 mg to 500 mg | from 0,012 mg to 0,014 mg | ЛКСИМ-М-02:2021 |
|  |  |  | from 1 g to 500 g | from 0,030 mg to 0,16 mg |  |
|  |  |  | from 1 kg to 5 kg | from 6,0 mg to 17 mg |  |
|  |  |  | from 10 kg to 20 kg | from 45 mg to 89 mg |  |
|  |  |  | 1000 kg | 27 g |  |
| 4. | Electronic scales with non-automatic action | Mass, kilogram kg and its submultiples and multiples | up to and including 2,2 kg | from 0,012 mg to 0,1 mg | ЛКСИМ-М-01:2019 |
|  |  |  | from 2,2 kg to 100 kg | from 0,1 mg to 200 mg |  |
|  |  |  | from 100 kg to 300 kg | from 0,2 g to 3 g |  |

Fixed Scope

References:

ЛКСИМ-М-01, Version 01/25.11.2019, Calibration procedure for electronic scales with non-automatic action

ЛКСИМ-М-02, Version 02/27.09.2021, Calibration procedure for weights and sets of weights by direct comparison method